Format SD card

- Run Win32Discimager.exe

- Select image file : 2017-01-11-raspbian-jessie.img

- Select “Device”

- Select “Write”

VNC viewer Setup

- Install tightvncserver   sudo apt-get install tightvncserver

- sudo apt-get install x11vnc

- Set VNC Server IP : 192.168.38.104:1

- Name : Any

Username : pi

Password: raspberry

====================================================================

- 如何在開機時就啟動 VNC 伺服器？

  - sudo nano /etc/init.d/tightvncserver

內容如下，重要的是在 start) 部份設定啟動的參數。

#!/bin/bash

### BEGIN INIT INFO

# Provides:          tightvncserver

# Required-Start:    $syslog

# Required-Stop:     $syslog

# Default-Start:     2 3 4 5

# Default-Stop:      0 1 6

# Short-Description: vnc server

# Description:

### END INIT INFO

export USER='pi'

eval cd ~$USER

# Check the state of the command - this'll either be start or stop

case "$1" in

start)

# if it's start, then start vncserver using the details below

su $USER -c '/usr/bin/vncserver :1 -geometry 800x600 -depth 16 -pixelformat rgb565'

echo "Starting vncserver for $USER "

;;

stop)

# if it's stop, then just kill the process

pkill Xtightvnc

echo "vncserver stopped"

;;

*)

echo "Usage: /etc/init.d/tightvncserver {start|stop}"

exit 1

;;

esac

exit 0

修改腳本權限。

pi@raspberrypi ~ $ sudo chmod 755 /etc/init.d/tightvncserver

將該腳本加入預設的啟動程序。

pi@raspberrypi ~ $ sudo update-rc.d tightvncserver defaults

重新啟動 Pi，再執行 netstat -nutlp 或是 ps aux | grep vnc，可以看到 VNC 伺服器已經跑起來了。

- 我們可以用 vncpasswd 這個指令改 VNC 的連線密碼

Raspberry PI(RASPBIAN) ftp伺服器安裝

- sudo apt-get install vsftpd
2.  sudo nano /etc/vsftpd.conf
3. 刪除前面的 “#"==> at line local enable =yes
  刪除前面的 “#"==>at line write enable =yes
  local_enable=YES, write_enable=YES
   sudo service vsftpd restart 服務重新啟動

安裝完成後，我們可以來查看是否安裝成功，可用下列指令。
netstat -tul | grep ftp

4. 使用ftp客端軟體去連線
登入者請用pi
PS:

- port :21
sudo nano /etc/ftpusers
在root前邊加#,註釋掉它
設定root密碼sudo passwd root
解除root鎖定sudo passwd –unlock root
重開機
這樣就可以用root用戶登錄了

Install Netbaens

$ mv ~/Downloads/netbeans-8.0.2-201411181905-javase.zip ~/
$ unzip netbeans-8.0.2-201411181905-javase.zip

然後執行，
$ netbeans/bin/netbeans

Install USB serial devices

- Lsusb    to list all USB device

- dmesg  prints kernel debug/log information:

- shows me details for a device:

udevadm info -a -n /dev/ttyACM0

- use Minicom Terminal Program

sudo apt-get install minicom

minicom -D /dev/ttyACM0 -b 115200

- use udev rules

  - In /etc/udev/rules.d I create a new file named:  99-usb-serial.rules

(The number 99 is because rules >=90 should run last)

  - To create a dedicated device for the Segger J-Link I can add the following line to the 99-usb-serial.rules:

SUBSYSTEM=="tty", ATTRS{idVendor}=="04d8", ATTRS{product}=="000a",

ATTRS{manufacturer}==" Microchip Technology Inc.",SYMLINK+="ttyS1"

  - That means that the device will be still used as \dev\ttyACMx, but a symbolic link /dev/ttyUSB50 or /dev/ttyUSB51 will be created.

  - use the following command to test the rules:

udevadm test /dev/ttyACM0

  - To test my new rules, I have to disconnect and re-connect the USB devices. or “ udevadm trigger”, “ls -l /dev”

  - I should see that a link has been created:

lrwxrwxrwx 1 root root           7 Dec 26 19:23 ttyUSB50 -> ttyACM0

lrwxrwxrwx 1 root root           7 Dec 26 19:30 ttyUSB51 -> ttyACM1

- Test  : install GtkTerm    sudo apt-get install gtkterm

  - Use gtkterm to send data to ttyACM0

Install Pi4J tool package

- Direct download from internate

curl -s get.pi4j.com | sudo bash

- Offline Manual

- Download from : http://get.pi4j.com/download/pi4j-1.1.deb

- transfer the download installer package over to your RaspberryPi

- sudo dpkg -i pi4j-1.1.deb

- upgrade :

sudo apt-get install pi4j or pi4j --update

- Installed Location / Example Files

/opt/pi4j/lib 
/opt/pi4j/examples

- compile a Java program using the Pi4J libraries,

javac -classpath .:classes:/opt/pi4j/lib/'*' –d . SerialExample.java

- to start a Java program using the Pi4J libraries

sudo java -classpath .:classes:/opt/pi4j/lib/'*' SerialExample

- pi4j Web resource  :

http://pi4j.com/example/serial.html

15#

NetBean IDE setup

Options  Java  Java Platform Manager  your java install version

- Auto startup setting 1 : startup from terminal

  - cd /home/pi

  - Sudo nano ~/.bashrc

  - Add  “bash myjar.sh” in the last line

PS:  SSH disabled by default; can be enabled by creating a file with name "ssh" in boot partition

Run GUI as root

sudo nano /etc/rc.local

Before the "exit 0" line in it add the following line:

#Auto run the GUI as root

sudo startx

Save it by pressing Ctrl+X, " Y", ENTER

Re-boot your RPi and it should automatically run the GUI as the root user.

- Setting An Application / Your Application To Automatically Run In The GUI

  - For the standard Pi user (if not using "sudo startx"):

sudo nano /home/pi/.config/lxsession/LXDE-pi/autostart

- For the root Pi user (if running the GUI with "sudo startx"):

sudo nano /root/.config/lxsession/LXDE-pi/autostart

- Add    @lxterminal   in the end of lines

- Free SD card space

df -h shows you your disk usage

sudo apt-get purge wolfram-engine

sudo apt-get clean
sudo apt-get autoremove

…which showed me I could free up 658 MB

sudo apt-get remove --purge libreoffice*

sudo apt-get clean
sudo apt-get autoremove

…which showed another large space saving of 253MB

rem

First thing after afresh install I purge off the SD card are:

wolfram-engine

scratch

minecraft-pi

sonic-pi

dillo gpicview penguinspuzzle oracle-java8-jdk openjdk-7-jre oracle-java7-jdk openjdk-8-jre

Nearly 2 GB of things I do not need!

- Resizing a partition within an image file

- rpi-clone is a shell script that will back up (clone using dd and rsync) a running Raspberry Pi file system to a destination SD card 'sdN' plugged into a Pi USB port (via a USB card reader).

- rpi-clone works on Raspberry Pi distributions which have a VFAT boot partition 1 and a Linux root partition 2.

- rpi-clone does not work with NOOBS.

- rpi-clone can clone the running system to a new SD card or can incrementally rsync to existing backup Raspberry Pi SD cards.

- During the clone to new SD cards, rpi-clone gives you the opportunity to give a label name to the partition 2 .so you can keep track of which SD cards have been backed up.

- Just stick a correspondingly named sticky label on each SD card you have and you can look up the last clone date for that cardin the rpi-clone log file /var/log/rpi-clone.

- If the destination SD card has an existing partition 1 and partition 2 matching the running partition types, rpi-clone assumes (unless using the -f option) that the SD card is an existing backup with the partitions properly sized and set up for a Raspberry Pi.

- This is done by a partial 'dd' from the running booted device /dev/mmcblk0 to the destination SD card /dev/sdN followed by a fdisk resize and mkfs.ext4 of /dev/sdN partition 2.

- This creates a completed partition 1 containing all boot files and an empty but properly sized partition 2 rootfs. The SD card partitions are then mounted on /mnt/clone and rsynced to the running system.

- This is so you can go look at the clone results or make any custom final adjustments if needed. For example, I have a couple of Raspberry Pis and I use one as a master. When I clone for the benefit of the second Pi, I do a "cd /mnt/clone/etc" and fix the files needed to customize for the second Pi (well, actually I do that with a script that takes my desired Pi hostname as an argument).

- Either way, you typically might need to change at least these files:

/etc/hostname           # I have one of rpi0, rpi0, ...

/etc/hosts              # The localhost line should probably be changed

/etc/network/interfaces   # If you need to set up a static IP or alias

- If you cd into the /mnt/clone/tree to make some of these customizations or just to look around, don't forget to cd out of the /mnt/clone tree before telling rpi-clone to unmount.

- Backup Raspberry SD by using rpi-clone

- rpi-clone is on github, to get it and install it to /usr/local/sbin:

Go to https://github.com/billw2/rpi-clone and download the zip file:

$ unzip rpi-clone-master.zip

$ cd rpi-clone-master

$ cp rpi-clone /usr/local/sbin

- or, use git to clone the repository:

$ git clone https://github.com/billw2/rpi-clone.git

$ cd rpi-clone

$ sudo cp rpi-clone /usr/local/sbin

$ cd /usr/local/sbin

$ sudo rpi-clone sda –f -v

- or operation from GUI

sudo chmod +x rpi-clone    to make the file executable

sudo mv rpi-clone /usr/local/sbin Open a terminal and move the rpi-clone script to the usr/bin folder

Now create a .desktop file at the appropriate location by typing...

cd /usr/share/raspi-ui-overrides/applications
sudo nano rpi-clone.desktop

...then enter the following...

[Desktop Entry]
Name=RPI-Clone
Comment=Clones the OS SD card to a USB-connected SD card of greater capacity than the total occupied file space
Icon=/usr/share/pixmaps/rpi-clone.jpg
Exec= lxterminal -e 'sudo /usr/local/sbin/rpi-clone sda -f -v'
Type=Application
Terminal=false
Categories=Utility

Note: The "Comment=" line should all be on one line.

You can change the jpgfile by yours in : /usr/share/pixmaps/rpi-clone.jpg

- Now look in Menu/Accessories and rpi-clone should be there ready for launch at the click of a mouse button whenever you need it.

- Load MyPi Server on boot:

- Stop MyPi Server by pressing CTRL C twice.

- Edit rc.local file by typing:

sudo nano /etc/rc.local

- Scroll down and type the following command BEFORE the exit 0 line:

python3 /home/pi/mypi_server.pyc &

https://www.raspberrypi.org/forums/viewtopic.php?f=29&t=137693&p=914109#p914109

- Raspberry Pi 解鎖 root 帳號

  - sudo passwd root

  - 會要求你輸入兩次新密碼，這時你可以輸入你想設的密碼。

  - sudo passwd -u root

  - 再輸入這個， root 帳號就解鎖了！

How to run MyPi TCP server

- Donwnload & install MyPi TCP Server

$Cd /home/pi

$wget www.ioslinks.com/mypi/server.zip

$unzip server.zip

There will hav mypy_server.pyc  &  mypi.cfg two file

- Run & Test MyPi Server

    - Start Mypi server by typing :

$python3 mypi_server.pyc

    - Then you can run IOS app

    - In mypi.cfg :

      - TCP port :5000

      - Connection Password : 123456

      - Output pin : 7,11,13,15,16,18,19,21

      - Input pin: 29,31,32,33,36,37,23,24

- Send data directerly to Printer

  - $ stty -F /dev/ttyACM0 115200

  - $echo -e "This is a test.\n" > /dev/ttyACM0

- install CUPS library

$ sudo apt-get update

$ sudo apt-get install libcups2-dev libcupsimage2-dev git build-essential cups system-config-printer

- install zj-58

$git clone https://github.com/klirichek/zj-58.git

$cd zj-58/

$make

$sudo ./install

  - lpt command

    - $lpstat –P      ask printer status

    - $echo "This is a test." | lpr

    - $lpr -o fit-to-page /usr/share/raspberrypi-artwork/raspberry-pi-logo.png

- List the devices

# lpinfo -v #

$ /usr/lib/cups/backend/snmp ip_address # Use SNMP to find a URI

- List the drivers

$ lpinfo -m

- Add a new queue

# lpadmin -p queue_name -E -v uri -m model

- The queue_name is up to you. Example:

# lpadmin -p HP_DESKJET_940C -E -v "usb://HP/DESKJET%20940C?serial=CN16E6C364BH" -m drv:///HP/hp-deskjet_940c.ppd.gz

# lpadmin -p SHARED_PRINTER -m raw    # Raw queue; no PPD or filter

- Set the default printer

$ lpoptions -d queue_name

- Change the options

$ lpoptions -p queue_name -l # List the options

$ lpoptions -p queue_name -o option=value # Set an option

- Example:

$ lpoptions -p HP_DESKJET_940C -o PageSize=A4

- Check the status

$ lpstat -s

$ lpstat -p queue_name

- Deactivate a printer

# cupsdisable queue_name

- Activate a printer

# cupsenable queue_name

- Set the printer to accept jobs

# cupsaccept queue_name

- Remove a printer

- First set it to reject all incoming entries:

# cupsreject queue_name

- Then disable it.

# cupsdisable queue_name

- Finally remove it.

# lpadmin -x queue_name

- Print a file

$ lpr file

$ lpr -# 17 file                # print the file 17 times

$ echo "Hello, world!" | lpr -p # print the result of a command. The -p switch adds a header.

- Check the queue

$ lpq

$ lpq -a # on all queues

- Clear the queue

# lprm   # remove last entry only

# lprm - # remove all entries

- Build  CAUPS UBUNTU16  printer driver

    - Modify WptCUPS4Ubuntu20170310.sh

      - Mark line 15 :  “rm -f /tmp/wptcups.tgz”

        - $sudo ./QuickConfig.sh

will generator the wptcups.tgz

        - Then you can modify/add files in the

      - Encode the tar file

        - $uuencode –m /tmp/wptcups.tgz wptcups.tgz > wptcups.tgz.base64

        - Copy the contain of the wptcups.tgz.base64 into the WptCUPS4Ubuntu20170310.sh

- Remove cups :

  - Sudo apt-get autoremove

  - $sudo clean

- Install cups

  - sudo apt-get install cups

  - sudo usermod -a -G lpadmin pi

  - sudo nano /etc/cups/cupsd.conf

  - modify the following :

# Only listen for connections from the local machine
# Listen localhost:631
Port 631

- install EPSON TM88 printer

- IOT printer (https://www.dfrobot.com/blog-407.html)

  - Install Neopixel

    - "sudo apt-get install python-pip"

    - "sudo pip install imapclient"

    - "sudo apt-get install python-serial python-imaging python-unidecode"

    - "sudo apt-get install git"

    - "git clone https://github.com/DAFRELECTRONICS/IoTprinter"

    - "cd IoTprinter"

    - "sudo python printertest.py"

  - Configuring Gmail account for the printer

  - Configuring the script in the Raspberry

    - Install neopixel

      - sudo apt-get install build-essential python-dev git scons swig

      - git clone https://github.com/jgarff/rpi_ws281x.git

      - cd rpi_ws281x

      - scons

      - cd python

      - sudo python setup.py install

    - “sudo nano mailverifier.py”

      - Need to preinstall “sudo pip install cryptography”

    - “sudo python mailverifier.py”

  - Need Auto run ?

    - sudo nano /etc/rc.local  add the following

cd /home/pi/IOTPrinter
python mailverifier.py &

- UBUNTU install java ( for PC)

$ sudo apt-get install default-jdk

$ sudo add-apt-repository ppa:webupd8team/java

$ sudo apt-get update

$ sudo apt-get install oracle-java8-installer

$ sudo update-alternatives --config java

$ sudo nano /etc/environment

Add  JAVA_HOME="/usr/lib/jvm/java-8-oracle"

$ reload       $ source /etc/environment

- UBUNTU fix Broken sudo

  - $ mount -o rw,remount /

  - $ adduser username sudo

- UBUNTU install CUPS

  - $ sudo apt install cups

  - $ sudo nano /etc/cups/cupsd.conf

        - Browsing On

  - $ sudo systemctl restart cups.service

  - $ sudo usermod –aG lpadmin winpos

- UBUNTU  /dev/ttyACM0 access denined solution

  - /etc/apparmor.d/usr.sbin.cupsdA   add “/dev/ttyACM* rw,

Install Linux CDC ACM driver

Install JAVA JDK8

- 下載JDK 8 for ARM

- $ sudo mkdir -v -v /opt/java
$ sudo tar zxvf ~/jdk-8u33-linux-arm-vfp-hflt.tar.gz -C /opt/java

- $ sudo update-alternatives --install "/usr/bin/java" "java" "/opt/java/jdk1.8.0_33/bin/java" 1
$ sudo update-alternatives --set java /opt/java/jdk1.8.0_33/bin/java

- 另外，有些軟體需要環境變數JAVA_HOME，請編輯~/.bashrc，加入：
export JAVA_HOME="/opt/java/jdk1.8.0_33"
export PATH=$PATH:$JAVA_HOME/bin

- Install and Setup Samba Server

https://eltechs.com/raspberry-pi-nas-guide/?utm_source=facebook.com&utm_medium=smm_post&utm_campaign=raspberry_pi_nas_guide

$ sudo apt-get update

$ sudo apt-get upgrade

$ sudo apt-get install samba samba-common-bin

open the /etc/samba/smb.conf and add the following:

[INTERNAL]

comment = internal Files

browseable = yes

path = /home/pi/INTERNAL

writeable = Yes

create mask = 0777

directory mask = 0777

browseable = Yes

public = yes

$ sudo smbpasswd -a pi   to change the smba password

$ sudo /etc/init.d/samba restart

On Windows open the Explorer and choose “Map network drive” from the Menu

選連線網路磁碟